TOYOTA ESP9100 NET Machine Operating System Update

A.  To verify the current Machine Operating System:

Place machine in test mode:

• On right side of control panel place DSW1-1 in on position • Turn machine ON in 'Test Mode.' • Press SET • Make note of your current machine specs

- To download the newest Operating System version 2.0 :

Note: These design downloads are in a zipped format. If you do not have a copy of WinZip to unzip this file, you may visit www.WinZip.com to download an evaluation version.

To download file:

• Make a folder on your computer's hard drive (such as C:\opersys) • Save the file to the new folder • Double-click to unzip the file • Place a thumb (or flash) drive in your computer's USB port • Once unzipped save the three files to your USB port • Once completed, remove the thumb drive and follow steps below

C.  To update the Operating System:

Step 1: Detach the switch cover by loosening the screw.

Updating Previous Version to 1.3

Step 2: Place switches: • DSW1-1 on. • DSW2-1 on..

Step 3: Set the program disk.

• Place the thumb drive in the embroidery machine's USB port.

Step 4: Turn ON the power switch at the power supply box.

Step 5: Select the program to be installed.

• Press the Right Arrow, followed by the Down Arrow. Repeat pressing the Right Arrow, followed by the Down Arrow 2 additional times (3 times total). The word "INSTALL" should appear next to "MAIN", "XY", and "PANEL".

Step 6: Verify current specs.

After update to version 2.0 is complete, your current specs should be: Panel; 2.0. Main; 2.0. XY; 2.0.

Step 7: Complete installation.

• Press Set • After install is complete (confirm "Install Complete" on display), power off machine, return all DIP switches to off and remove the thumb drive.